Design and Implementation of Amplitude Modulation (AM) Transmitter and Receiver System

1.Basic description of idea

The goal of this project is to design and implement a compact, self-contained Analog Communication Demonstrator Board focused on Amplitude Modulation (AM). The board integrates the complete AM transmission and reception chain onto a single platform.

I. Fundamental Concept

Amplitude Modulation (AM) is the process where the amplitude of a high-frequency carrier signal (Vc) is varied linearly in proportion to the instantaneous amplitude of a low-frequency message signal (Vm). The frequency and phase of the carrier remain constant. This technique enables the efficient, long-distance transmission of the low-frequency information.

where is the modulation index.

II. Core Functional Stages

The demonstrator board is divided into five distinct, testable modules:

1. Signal Generation:
Creates the two essential inputs:

Message Signal (Vm): A low-frequency (AF, e.g., 1 kHz) sine wave, externally applied from a function generator.

Carrier Signal (Vc): A high-frequency (RF, e.g., ~14 kHz) sine wave generated by a Hartley Oscillator using an LC tank circuit with inductors and a capacitor.

2. Amplitude Modulator:
This core stage, implemented using a BJT (BC547B), combines the carrier and message signals to produce the Amplitude Modulated (AM) waveform.
The transistor operates in its nonlinear region, where the instantaneous amplitude of the carrier varies in proportion to the message signal, creating the AM signal.

3.  Coupling and Low-Noise Amplifier Stage:

The modulated output from the transistor passes through coupling capacitors and biasing resistors to ensure DC isolation and maintain waveform fidelity.
It is then fed into a low-noise amplifier stage, which strengthens the AM signal while suppressing unwanted noise and interference, ensuring clear demodulation at the next stage.

4. Demodulation / Detector:
The AM signal is fed into a diode detector circuit (1N4148) followed by an RC filter network, which extracts the envelope of the modulated wave.
This process removes the high-frequency carrier component and recovers the original low-frequency message signal.

2.Methodology Undertaken

1.  Concept Understanding & Literature Review

- Studied AM theory, modulation index, carrier generation, and demodulation techniques.

- Reviewed BJT amplifiers and oscillator design concepts from the ECA syllabus (small-signal and large-signal transistor models).

2.  Design Stage

- Divided the project into four main functional blocks:

  - Signal generator(Hartley Oscillator)

  - AM Modulator (BJT-based)

  - Low-Noise Amplifier (Intermediate stage)

  - Envelope Detector (Demodulator)

3. Component Selection & Calculation

- Selected suitable component values using design formulas for desired carrier frequency and gain.

- Verified theoretical operating points (biasing, frequency response, gain).

4.  Hardware Implementation

- Built and tested each block separately on breadboard.

- Verified waveforms using CRO for correctness before integration.

5.  Integration & Testing

- connected all stages sequentially to form the full AM system.

- Checked the AM waveform at each test point and fine-tuned bias resistors for clarity.

  - Documentation & Report Preparation

- Compiled results, circuit diagrams, calculations, and theoretical background into the final lab report

3.Table of components used along with their specifications

| Section | Component / Type | Specification / Description | Purpose / Function |
| --- | --- | --- | --- |
| A. Power Supply | DC Supply | +12 V regulated | Provides operating voltage for all active stages |
| B. Active Devices | Transistors | BC547B (×2) | Used in oscillator/modulator and audio amplifier stages |
| C. Hartley Oscillator (Carrier Generator) | Inductors (L₁, L₂) | ≈ 6.5 mH each (or tapped coil) | Forms LC tank for carrier generation |
|  | Capacitor (Cₜ) | ≈ 0.01 µF | Sets carrier frequency with L₁, L₂ |
|  | Bias & Load Resistors | kΩ range | Provide transistor bias and collector load |
|  | Coupling Capacitor | 0.01 µF (typ.) | Couples carrier output to modulator |
| D. Modulator Section (AM Generation) | Audio Input | 1 kHz sine wave | Message signal for modulation |
|  | Carrier Input | ~14 kHz from oscillator | High-frequency carrier signal |
|  | Coupling Network | R-C network | Mixes carrier and message signals for AM output |
| E. Receiver / Demodulator | Diode | 1N4148 | Envelope detection of AM signal |
|  | Resistor & Capacitor | 10 kΩ, 0.1 µF (typ.) | RC filter to recover audio waveform |
| F. Audio Amplifier | Transistor (BC547B) | – | Amplifies recovered audio |
|  | Bias & Coupling Components | kΩ resistors, µF capacitors | Signal coupling and gain stabilization |
| G. Output Stage | CRO / Speaker | – | Displays or outputs recovered audio signal |

4. Input and Output with Justification

| Stage | Input | Output | Expected Behaviour / Justification |
| --- | --- | --- | --- |
| Hartley Oscillator | DC supply voltage (+12 V) and LC tank circuit components (L₁, L₂, C) | High-frequency carrier signal (100 kHz – 1 MHz) | Generates stable sinusoidal carrier waveform using inductive feedback from LC tank circuit |
| AM Modulator | Audio (1 kHz sine wave) + Carrier (from Hartley oscillator, 100 kHz – 1 MHz) | AM waveform (carrier amplitude varies) | Envelope follows the audio waveform, confirming proper amplitude modulation |
| Low Noise Amplifier | AM signal | Stronger AM signal | Amplifies the modulated signal without altering its waveform (no distortion) |
| Envelope Detector | AM signal | Recovered audio waveform | Diode detector extracts the signal envelope, reproducing the original low-frequency message |

5.Calculation

6.Plot of output with discussion

Plot 1: AM Modulated Signal (Modulator Output)

Print Out: [Attach CRO printout of the AM waveform showing carrier with varying envelope.]

Discussion:
The plot confirms amplitude modulation with the envelope varying according to the message signal. The modulation index, calculated from envelope peaks, was maintained within a suitable range, avoiding distortion. The carrier frequency and phase remain stable, matching the expected AM waveform equation.

Plot 2: Recovered Message Signal (Demodulator Output)

Print Out: [Attach CRO printout of the envelope detector output, ideally overlaid with the original input.]

Discussion:
This plot shows the recovered message signal with minimal RF ripple, confirming effective envelope detection. The RC time constant satisfies the condition to remove the carrier while accurately following the message envelope, validating the theoretical analysis.

7.Student contribution